3013 SERIES

DIAPHRAGM

COMPRESSORS & VACUUM PUMPS

NOTE:  Model 3003 will be Superseded by Model 3013 in Year 2001

FEATURES:

- Lightweight:  40g

- Small Size:  1.4” x 0.9” x 1.7” - 36mm x 22.5mm x 42mm

- Motor voltage variations available to suit most OEM applications

- Standard DC motors:  4.5, 6, 12VDC (12V 	preferred)

- LC motors:  3.0VDC, 4.5VDC, 12VDC

- BLDC:  6V BLDC

- Motor Types Available:  DC, BLDC

- Free Air Flow:  760 ml/minute up to 1.2 lpm

- Maximum Pressure:  8 PSIG (550 mbar)

- Maximum Vacuum:  13.5” Hg (450 mbar); 12” Hg (400 mbar) typical max. vacuum

- Maximum Power Consumption: DC:  0.5 - 1.0 watt

- Stroke Selection Available:  Yes, 95% std. 0.7mm, 0.5, 0.9mm

- Diaphragm Materials Available:  EPDM std.; Viton

- Valve Materials Available:  EPDM std.; Viton

- Design Life: 500 – 1,000 hours LC motor; 1,000 - 3,000 hours std.; BLDC 10,000+ hours

- Sound Level:  Approx. <45 dB(A) typical

- Port Locations:  D std; A optional

- Start against pressure/vacuum possible

best suited for the following applications:

- Portable Gas Detectors

- Blood Pressure Devices

- Breath Analyzers/Anesthesia Devices

- Pipettes

- Air Samplers

- Particle Counters

- Gas Analysis

Potential competitors/models (if known):

- Gast Model 5D

- KNF Neuberger Model UNMP08

- T-Squared Model T2-03 Compact

- Hargraves Technologies Corp. Model CTS

- Sensidyne Model A

- Oken Seiko

- Schwarzer (MPM) Model 500 EC